MIIA RISK MANAGEMENT ADVISORY

STATIONARY COMPACTORS and SITE SAFETY AT TRANSFER STATIONS

# Hazards - Severe fall, crushing, and caught between hazards are posed when employees or contractors:

- Feed materials into compactor.

- Attempt to clear jams and retrieve items that fell in.

- Clean, repair, or maintain compactor.

- Attempt to use the compactor opening with unprotected edges.

Fatality Assessment and Control Evaluation (FACE) – A NIOSH report of 20 Compactor Related Fatalities found that victims had either reached or fell into the compression chamber, and were crushed in the compacting chamber, or suffered amputation from being caught between the ram and the compacting chamber walls.

These fatal incidents illustrate a lack of policy, administrative controls, and work practices to prohibit entering the compactor as the victims had been clearing a jam or were attempting to retrieve material. Other cases illustrate a lack of Lock-Out/Tag-Out and Permit Required Confined Space Programs as the machine cycled automatically when victim entered compacting chamber, and when coworkers activated compactor without knowing a worker was inside.

# Risk Reduction Strategy Recommendations:

- Prohibit bypassing the interlock on compactors and operating them with the doors open.

- Prohibit entry into the compactor for any reason, unless trained and authorized to do so:

  - Post a Permit Required Confined Space sign near the compactor(s).

  - Require contractors to have a Lock-Out/Tag-Out (LO/TO) Program or implement a LO/TO Program for all work requiring deenergizing the compactor.

  - Provide tools, protocols, and training for retrieving items and clearing jams without entering the compactor.

  - Provide guardrails for each type of unprotected openings:

Per OSHA specifications for access to the compactor by pedestrians.

Per OSHA specifications to resist a vehicle for breaching compactor openings by vehicles.

Site Safety

- Hazards - people have been hit due to inadequate visibility and lack of traffic and site controls on a site with moving vehicles and pedestrian and employee foot traffic.

- Risk Reduction - provide speed limits, signage, traffic control devices ( e.g. cones, barriers), pavement markings, etc. for traffic control, and adequate lighting and high visibility vests for employee visibility.

© MIIA All rights reserved. Created April 8, 2024, Revised December, 2024

Massachusetts Interlocal Insurance Association | Serving Massachusetts’ communities since 1982 Boston, MA | 617-326-7272 | 800-882-1498 | www.emaiia.org